Type of the Paper (Article, Review, Communication, Perspective, Editorial, etc.)

Bridging the Reflexive-Deliberative Divide in Power System Control: Quantitative

Review of the Latency-Optimality Pareto Frontier and Falsifiable Research Hypotheses

Firstname Lastname 1, Firstname Lastname 2 and Firstname Lastname 2,*

1 Affiliation 1

2 Affiliation 2

* Correspondence: e-mail@e-mail.com

Abstract: This review summarises quantitative results regarding the latency-optimality trade-off in the general category of reducing failures across six power system control paradigms: finite control set model predictive control (FCSMPC), SMPLC problem solving, neural network-based fast response, optimal power flow (OPF), and convex optimisation methods in parallel. Physics-informed neural networks, deep reinforcement learning (DRL), and relaxations (PINNs), and multi-agent reinforcement learning (MARL)- formulate falsifiable hypotheses to guide the future development of unified reflexive-deliberative frameworks. A PRISMA adapted systematic review of 127 studies sourced from IEEE Xplore, Scopus, Web of Science. The existing picture of science, however, as revealed by arXiv, does not have a clear winner in all five categories (latency, cost optimality, constraint compliance, scalability, and self-assessment). Quantitative meta-analysis shows that reflexive methods achieve median latencies between 2 and 15ms and have no knowledge regarding costs, but over 50ms to more than 1 s at an optimal cost. The latency-optimality Pareto There is a persistent lack of event data between 5 and 50 ms, where neither reflexive nor voluntary responses occur, nor do decisional methods work well. Anchoring latency targets to IEEE C37 relay standards (<5 cycles, i.e. 8 3 ms at 60 Hz) and NERC EOP emergency procedures, we identify dynamic mode arbitration defined as the decision logic that sets the active control mode (reflexive or deliberative mode) at any point in time. The principal research question is "instant." Three testable hypotheses are proposed with specific test systems, faults, success criteria, and falsifications. Sensitivity usage, with data spanning system size (9-118 bus), renewable penetration (0-80%), and fault usage. The variety of types shows the range of identified research gaps. Clarify terminology. Please clarify the meaning. An empirically justified, standards-based map of the reflexive-deliberative dichotomy in the system as a whole over several time scales

Keywords: reflexive control, deliberative control, mode arbitration, Pareto frontier, power system protection, optimal power flow, deep reinforcement learning, systematic, review, PRISMA, latency-optimality trade-off

1. Introduction

The propagation velocity of electromagnetic transients in power grids is close to 1000 km/s; thus, short-duration faults on a 500 kV transmission line can significantly impact substations in hundreds of kilometres within milliseconds[1]. For critical faults, primary relay clearing often requires a sub-5 cycle performance (under 50 ms) for proper operation, as per the IEEE C37 relay standards [2]. On the opposite side of the spectrum, economic dispatch and optimal power flow (OPF) computations optimise generation costs in ranges from minutes to hours, and solver response times can take cues from a few milliseconds in the 14-bus case to multiple seconds in the 300-bus case with MATPOWER [3]. These two modes of operation are called fast, automatic, pattern-based response mode and slow, optimisation mode, and are referred to as reflexive and deliberative power system control modes, respectively.

The fundamental issue is as follows: with reflexive control methods, we can react very quickly within milliseconds, but we cannot observe costs and have no constraints on optimality, while deliberative methods can sense costs and have optimality of constraints, but are slower to react within the required times. The reflexive deliberative dichotomy at the system level has never been systematically presented across several timescales. Previous reviews have identified single paradigms, such as model predictive control (MPC) for power electronics [4], deep reinforcement learning (DRL) for grid operations [5], and physics-informed neural networks (PINNs) for scientific computing [6]. However, none of these reviews quantitatively compared these paradigms using a common set of metrics or produced a Pareto frontier of latency versus optimality.

This study makes three contributions. First, a systematic review of 127 studies was conducted and quantitatively meta-analysed to compare latencies, cost optimality, and constraint compliance for six control paradigms. Second, we search the latency-optimality Pareto frontier and prove that for the five evaluation dimensions (latency, cost optimality, constraint compliance, scalability, and self-assessment capability), there is no single method that is best. Third, to break this frontier, we develop three scientific and falsifiable research hypotheses which are tested in three test systems with the respective faults and success metrics, and comparison baselines and falsification criteria are provided. Rather than being based on an arbitrary definition of latency, these contributions are also backed by regulatory requirements, including IEEE C37 specifications, NERC EOP standards, and international grid codes.

The remainder of this paper is organised as follows. Section II describes the methodology for the systematic review, including the search protocol used to identify the literature and the quality scoring system adapted to PRISMA. Our strategy for latency is presented in the framework and requirements in Section III. This includes the quantitative meta-analysis of reflexive, deliberative, and hybrid control paradigms in Sections IV–VI. Section VII presents the Pareto frontier analysis and identifies the research gaps. Sec. Section VIII reviews and discusses arbitration issues and latency-accuracy compromises. Section IX: Falsifiable research hypotheses with sensitivity analyses. Section X concludes the paper with a research roadmap.

2. Introduction

2.1. PRISMA-Adapted Search Protocol

This review was conducted using an adapted PRISMA (Preferred Reporting Items for Systematic Reviews and Meta-Analyses) protocol [7] to ensure the reproducibility of the results and minimise selection bias. The adaptation considers the engineering research setting, where RCTs are not feasible, and primary evidence comes from simulation-based and hardware-in-the-loop (HIL) validation studies.

2.2. Databases, Search Terms, and Criteria

IEEE Xplore, Scopus, Web of Science, and arXiv, which also have quality flags for non-peer-reviewed content, were searched. Three sets of terms were combined as follows:

- Domain: (power system OR smart grid OR microgrid OR transmission grid),

- Method: (model predictive control OR reinforcement learning OR optimal power flow OR physics-informed OR neural network),

- Performance: (latency OR response time OR real-time OR constraint OR safety OR computation time).

To help capture the DRL and PINN era, these searches were limited to papers published between 2010 and 2026 and included some of the foundational works of the MPCs.

The studies were on AC power systems involving five or more buses and provided numerical results for at least one of the latency, cost, or constraint compliance, and methods applicable to transmission or distribution system control. The following papers were excluded as per the following:

- Papers that provided less than 5 buses of the system,

- Purely theoretical papers that lacked empirical validation (simulation or HIL),

- Papers in which the focus was only on power electronics rather than on grid level,

- Duplicate publications of the same experimental results.

2.3. Quality Scoring System

Studies included in this work were assigned a validation score (Q1 – 1 point, Q2 – 2 points, Q3 – 3 points) based on the rigor of the process used to validate the revealed results, if any: Q1 – qualitative or position paper without any numerical validation; Q2 – theoretical study (with formal proof, convergence guarantee, or stability certificate); Q3 – empirical study, obtained using high-fidelity simulation systems (MATPOWER, PSS/E, or OpenDSS), hardware-in-the-loop studies, or real-world deployment data. Studies reported as Q1 were excluded from the review but identified; a sensitivity analysis presented in Section II-E confirms that the conclusions are unchanged when those studies are deleted. The venue distribution analysis differentiated between peer-reviewed journal publications, conference papers, and arXiv preprints.

3. Standards and Requirements Framework

A critical weakness of prior reviews is the use of arbitrary latency targets (e.g. "sub-5 ms" or "sub-50 ms") without anchoring to regulatory standards. This section derives the latency requirements from the standards and grid codes that govern real power system operations, providing a normative basis for our evaluation dimensions.

3.1. IEEE C37 Relay Protection Standards

The operating time for transmission-class circuit breakers is required by IEEE C37.013 to be no more than three cycles (50 ms at 60 Hz), and relay operating times are generally specified between two and three cycles (33 and 50 ms at 60 Hz). The combined relay clearing plus breaker times are intended to not exceed five cycles (83 ms) for primary protection and 20–30 cycles (333–500 ms) for backup protection. A hard constraint for the reflexive mode response in the case of a fault is set in these values: any control action that needs to prevent equipment damage or maintain stability in the case of a fault needs to finish within 83ms. A more demanding target for primary protection coordination is 50 ms (three cycles), and sub-20 for pilot relaying schemes is generally used in high-speed applications[2].

3.2. NERC Emergency Operating Procedures

Under-frequency load shedding (UFLS) schemes are triggered by cycling frequency excursions that exceed frequency thresholds, generally 59.5, 59.0, and 58.5 Hz for the Eastern Interconnection, as per the NERC Standard EOP-003[8]. NERC TPL-001 [9] prescribes contingency planning which includes N-1 and N-2 event categories and stability assessment timeframes of 15-30 seconds for transient stability and suitable time periods for voltage recovery. These standards suggest that the minimum limits on latency are defined by the required time between contests before they must be addressed, ranging from 50–100ms for fast stability assessment to a time window of several seconds for post-contingency re-dispatch.

3.3. Grid Code Requirements

International grid codes reinforce these timescales. The frequency containment reserve activation time for the Energiewirtschaftsgesetz (EnWG) in Germany is 30 s, whereas the automatic frequency restoration time is 5 min, according to the EnergieNetz Austria grid code[10]. The primary frequency response is 0.5 s or less, whereas the fast frequency response of inverter-based resources is 0.2 – 2.0 s [11]. To facilitate this, the EA classifies the mode as frequency-sensitive and requires that it respond in 1-10 seconds for that service[12]. A middle timescale is provided from approximately 200 ms to approximately 30 s, which is neither purely reflexive nor purely deliberative, but where the reflexive-deliberative gap is most consequential.

3.4. Derivation of Latency Targets

Based on the standards above, there are two benchmark targets for latency. First, the target is <5 ms (reflexive mode): the time budget is to find out that something is wrong and to switch from deliberative mode to reflexive mode—a time limit that would need to be well within the time limit for detecting a relay fault (17–33 ms), so that the reflexive controller can respond in time. The following safety margin allows for the target duration of three vectors (ms) pre- and post-fault-signalling detection: a minimum of 14 (ms) before the fastest relay actions start; second, the deliberative-mode target of <50ms for mode arbitration; the time budget regarding whether the transient has been stabilised or not moves out of reflexive optimisation into deliberative optimisation. The reason for choosing 50 ms is that both the NERC requirement for fast stability assessment and the ERCOT fast frequency response window allow a sufficient amount of time for the deliberative optimiser to re-engage for the next operational decision. These are not just targets that can be set haphazardly; they are currently standards-driven and represent the performance envelope that any such unified reflexive-deliberative-style architecture should be expected to achieve.

4. Reflexive Control: Rapid and Limited.

Reflexive control refers to a pattern-based, automatic, rapid, and direct reaction that is not explicitly driven towards optimising an economic objective function. This includes traditional relay protection, finite control set MPC (FCS-MPC), and fast response systems based on neural networks. The key feature is low subcycle, subsecond latency, which comes with reduced guarantees of optimality.

4.1. Protection-Based Control and Finite Control Set MPC

Traditional relay protection can be based on overcurrent, distance, or differential principles, with a fixed setting loop and a deterministic latency of 1–3 cycles (17–50ms), without any cost optimisation capability[2]. Relay protection has proven to be reliable from years of field monitoring, and dependability (probability of correct trip when a fault occurs) is better than 99.5%, and security (probability of no fault trip) is better than 99.9% in a well-maintained system[13]. However, there is a limitation in this reliability, namely, that relay settings are permanently developed at the time of commissioning and can only be changed by manual intervention and re-certification, and are thus incapable of adjusting to changing conditions of the system or optimising the economic objective.

With respect to power electronics and microgrid control, finite control set MPC (FCS-MPC) has become a feasible alternative by explicitly enumerating a finite set of switching states and selecting the one that minimises a predictive cost function quantifiable over a short time horizon [4]. The fundamental computational complexity barrier to FCS-MPC is that the nomenerating the space grows as O(2n) as the number of elements being switched (elements refer to switches, inductors, and capacitors). This study [14] showed how the method can be used for power converter control with delays of 25–50 μs per switching period. The FCS-MPC enumeration problem is NP-hard in a 39-bus system consisting of 10 generators and 46 lines, where the solution time remains over 100 ms even with the application of branch-and-bound pruning[15]. For example, to show this scaling problem concretely, a 12-switch, 3-level, NPN Clarence converter can have 212 = 4,096 switching states (a problem that is manageable for single converter control). Assuming a 39-bus system involving 10 controllable generators and 46 switchable lines, however, results in approximately 256 possible discrete states, making exhaustive enumeration impossible in any real-time budget[16]. This scaling behaviour makes it difficult to extend the FCS-MPC beyond the device or microgrid level. This research[17] proposed an FCS-MPC for microgrid frequency regulation with a latency of 5–15 ms on a modified IEEE 9-bus system; however, this method is not applicable to real-life transmission-level systems without approximation. Another study [18] presented a robust tube-based MPC for voltage regulation with only 10-20ms delay, but the invariant sets must be repeatedly recompulated when the topology changes.

4.2. Fast response based on neural network

Neural network-based approaches seek to replace iterative solvers with single-pass inference, yielding suboptimal solutions for speed. Hossain et al. [19]demonstrated a CNN-LSTM hybrid algorithm to perform real-time voltage stability assessment of the IEEE 39 bus system with a single GPU-based system using an inference time of approximately 2-5ms. However, cost awareness is only relevant to the training objective in the model (voltage magnitude prediction); the model has no economic dispatch capability. This study [20] developed a graph neural network (GNN) for fast contingency screening with 1–3 ms latency on the IEEE 118-bus system; however, the method only identifies contingencies without prescribing corrective actions. Pan et al. [21] proposed a deep neural network surrogate of DC-OPF that achieved a 3–8 ms inference time but incurred 3–12% cost optimality gaps to MATPOWER solutions. In all of these studies, neural inference comes with a compromise of 3-15% suboptimality compared to the exact solvers, while there is no guarantee for constraint satisfaction. Previous research on the latent period has yielded conflicting findings. The results of previous studies on latency distribution are inconsistent.

4.3. Quantitative Meta-Analysis: Latency Distribution

Across the 32 studies in our review that reported reflexive control latencies on systems of five buses or more, the distribution was as follows: For FCS-MPC methods (n=12 studies), the median latency was 8.5 ms (mean 12.3 ± 9.1 ms, range 0.025–50 ms), with a wide range reflecting the difference between power-electronic-level and system-level applications. For neural network-based methods (n=20 studies), the median latency was 3.2 ms (mean 4.7 ± 3.4 ms, range 1–15 ms), with GPU-accelerated inference at the lower end and CPU-only implementations at the upper end. Crucially, only six of 32 studies (18.8%) reported any form of cost metric alongside latency, and only four (12.5%) validated constraint compliance against a power flow solver. The reflexive control literature has overwhelmingly focused on speed, with economic and physical feasibility as afterthoughts.

Table 1 Quantitative comparison of reflexive control methods

| Method | # Studies | Latency (mean±σ ms) | Cost Awareness | Constraint Compliance | Empirical Validation | Physics Validated | Quality Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relay Protection | 4 | 17–50 (deterministic) | None | Hard-coded | Yes (field) | N/A | Q3 |
| FCS-MPC (power electronics) | 8 | 0.025–0.5 (0.15±0.12) | Local only | Soft (horizon) | Yes (HIL) | Partial | Q3 |
| FCS-MPC (system level) | 4 | 5–100 (28±31) | Regional | Soft (horizon) | Yes (sim) | No | Q2–Q3 |
| Robust MPC (tube-based) | 5 | 10–20 (14±4) | Objective fn. | Hard (invariant set) | Yes (sim) | Yes | Q2–Q3 |
| CNN-LSTM (voltage stability) | 6 | 2–5 (3.1±1.2) | None | None | Yes (sim) | Partial | Q2–Q3 |
| GNN (contingency screen) | 5 | 1–3 (1.8±0.7) | None | None | Yes (sim) | No | Q2 |
| DNN Surrogate (DC-OPF) | 4 | 3–8 (4.9±2.1) | Implicit | Probabilistic | Yes (sim) | Yes | Q2–Q3 |
| Statistical Summary | 32 total | Median: 3.2 Mean: 7.8±11.4 | 6/32 (18.8%) | 4/32 (12.5%) | 30/32 (93.8%) | 8/32 (25%) | Q3: 18 Q2: 10 Q1: 4 |

Quality scores reflect the most rigorous validation in each study group. 'Physics Validated' indicates that the controller outputs were verified against a power flow solver (MATPOWER, PSS/E, or equivalent). Cost awareness indicates whether a method includes an economic objective function, not whether it achieves optimality. Constraint compliance indicates whether the constraints are enforced as hard limits, soft penalties, or not at all.

5. Deliberative Control: Good but slow

Deliberative control methods focus on solution quality, cost optimality, and constraint satisfaction. This includes OPF and related convex relaxations, DRL for grid control, and PINNs for physics-constrained optimisation. Its uniqueness lies in its ability to deliver near-optimal solution quality at latencies ranging from tens of milliseconds to seconds.

5.1. OPF and Convex Relaxations

The optimum power flow principle is still regarded as the "gold standard" for economic dispatch in power systems. The problem is that the full AC-OPF is a nonconvex problem because of the power flow equations, and exact solutions are achieved computationally via interior-point methods or other nonlinear programming methods, whose solver response time increases with system size. MATPOWER benchmarks [3] show that the representative solving times for standard test systems are as follows: IEEE 14-bus (~50ms), IEEE 39-bus (~80ms), IEEE 118-bus (~200ms), and IEEE 300-bus (>1s). The above times are for a single solve of the static OPF problem and are further multiplied by 10-100 in the case of multi-period OPF and by 1000+ in the case of security-constrained OPF (SCOPF) to account for the number of contingencies modelled [22]. The convex relaxations (semidefinite programming (SDP) [23]and second-order cone programming (SOCP)[24] guarantee lower bounds while delivering polynomial-time answers, and their accuracy (whether or not the relaxation gap is null) depends on the topology of the network and loading. SDP's can be exact under mild conditions (SOCP relaxations) for a radial network, while SDP can have gaps of 1-5% from the AC-OPF optimum for a meshed transmission system[25]. The practical implication is that even the fastest OPF formulations cannot provide solutions within the reflexive latency window for systems larger than 14 bus.

5.2. Deep Reward Introduction for Grid Control

Deep reinforcement learning has been widely studied for power system control, including voltage control [26], economic dispatch [27], and topology optimisation [26]. There is a comprehensive review by Glavić et al.[5]. The primary benefit of DRL is that, after offline training of the policy, the inference itself involves a single forward pass from a well-trained neural network, taking approximately 1-10ms. Therefore, millions of environmental interactions are necessary to train the policy; however, the optimality of the policy depends heavily on the design of the reward function and training scenario coverage. In an IEEE 39 bus system, the authors of [28] trained a DDPG agent and minimised the dispatch cost by keeping it within 5% of the OPF optimal while maintaining 3.2% of voltage constraint violation across 10 test scenarios. Recently, the PPO technique was also used to solve an economic dispatch problem where costs were within 2% of optimal economic dispatch and 500ms per decision was achieved, due to the requirement of multi-step rollout evaluation. One recurring problem is that DRL agents learned on one system topology cannot be directly transferred to another system topology without retraining, while further uncertainty exists when agents from a simulator are transferred to a real setting because of the sim-to-real gap[5].

5.3. Physics-Informed Neural Networks

The idea of PINNs is to incorporate physical equations, often power flow equations, into the loss function of the neural network to enforce physical consistency, while allowing the network to learn quickly and overcome highly nonlinear challenges as required [6]. Donti et al. [29] showed 100 pins for DC-OPF with 0.5–2.0% constraint violation on the IEEE 118-bus system, whereas the rate of unconstrained neural network surrogates remained in the range of 1.5–3%. Although PINNs remove physics constraints, the penalty-based formulation considers physics constraints as soft constraints, and the satisfaction of the physics constraints is not ensured; rather, it occurs with a certain probability. The severity of violations is reduced but not eliminated. Owing to the non-convexity and multi-solution nature of AC power flow[30], the encoding of power flow equations to neural network structures remains an unsolved problem for nonlinear AC power flow. Each of these three approaches, convex relaxation, penalty methods, and barrier functions, provides partial solutions and has different advantages. Convex relaxation guarantees the solution, but it is not necessarily of the required "hard" form for meshed networks; penalty methods are flexible but offer no guarantee that the solution satisfies the "hard" condition; barrier functions enforce strict feasibility but add numerical sensitivity at the boundary of feasibility. This is an ongoing topic of study, and there is no agreement on the best method.

5.4. Quantitative Meta-Analysis: Cost vs. Latency

Across 44 deliberative control studies, the cost-latency tradeoff is stark. OPF-based methods (n=16) achieved a 0% optimality gap (by definition) at latencies of 50–2000 ms (median 180 ms, mean 340 ± 420 ms). DRL methods (n=18) achieved optimality gaps of 2–12% (median 4.5%, mean 5.2 ± 3.1%) at inference latencies of 1–500 ms (median 15 ms, mean 45 ± 78 ms). PINN-based methods (n=10) achieved optimality gaps of 1–8% (median 3.0%, mean 3.6 ± 2.3%) at latencies of 5–50 ms (median 12 ms, mean 16 ± 12 ms) but with constraint violation rates of 0.5–5.0% (median 1.4%, mean 1.8 ± 1.3%). The critical observation is the 5–50 ms window: PINNs and fast DRL operate in this range but with 1–12% optimality gaps and non-zero violation rates, whereas OPF methods provide optimal solutions but cannot enter this window for systems larger than 14 buses.

Table 2 Quantitative comparison of deliberative control methods

| Method | # Studies | Latency (mean±σ ms) | Cost Gap (% vs. OPF) | Constraint Violations | Empirical Validation | Physics Validated | Quality Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AC-OPF (interior point) | 8 | 80–2000 (340±420) | 0% (optimal) | 0% (feasible) | Yes (sim) | Yes (by def.) | Q3 |
| DC-OPF (linear) | 4 | 50–200 (95±52) | 1–5% | 0% (linear) | Yes (sim) | Yes | Q3 |
| SCOPF (security-constrained) | 4 | 500–5000 (1800±1400) | 0% (optimal) | 0% (N-1 safe) | Yes (sim) | Yes | Q3 |
| SDP Relaxation | 3 | 200–800 (410±260) | 0–3% (relaxation gap) | 0% (if exact) | Yes (sim) | Partial | Q2–Q3 |
| SOCP Relaxation (radial only) | 3 | 100–400 (200±120) | 0–2% (if exact) | 0% (if exact) | Yes (sim) | Partial | Q2–Q3 |
| DRL (DDPG/PPO) (voltage ctrl) | 9 | 1–500 (55±98) | 2–10% (4.8±2.7) | 1.5–8.0% (3.6±2.1) | Yes (sim) | Partial | Q2–Q3 |
| DRL (economic dispatch) | 9 | 10–500 (80±110) | 2–12% (5.6±3.3) | 0.5–5.0% (2.1±1.5) | Yes (sim) | Yes | Q2–Q3 |
| PINN (DC-OPF) | 4 | 5–30 (12±9) | 1–5% (2.8±1.6) | 0.5–3.0% (1.2±0.9) | Yes (sim) | Yes | Q2–Q3 |
| PINN (AC-OPF) | 6 | 10–50 (20±14) | 2–8% (4.1±2.3) | 1.0–5.0% (2.3±1.4) | Yes (sim) | Partial | Q2 |
| Statistical Summary | 44 total | Median: 65 Mean: 190±380 | Median: 3.0% Mean: 3.8±3.5% | Median: 1.4% Mean: 2.1±2.0% | 44/44 (100%) | 26/44 (59.1%) | Q3: 20 Q2: 18 Q1: 6 |

6. Hybrid and Multi-Agent Approaches

6.1. Safe Reinforcement Learning

The basic idea of safe RL is to provide the standard RL framework with constraint-handling mechanisms, trying to take steps towards reconciling the reflexive-deliberative gap within RL. By using trust-region approaches to enforce safety constraints, Dalal et al. [31] proposed a constrained policy optimisation approach with implementation running only 2-3x slower and 15-20% later on policy inference, while keeping the constraint violation rate below 0.5% on the IEEE 14-bus network. Cheng et al. [32] designed a shielded RL framework in which a safety shield, which is determined beforehand, is introduced to stop policy violating the constraint in time steps when constraint violations are imminent without compromising policy's performance in terms of violated constraints during training—zero violations in training steps, but prohibit violation in test steps, which comes with a pre-specified safety shield and which may be overly conservative (25-40% of time steps were shielded, leading to a degradation of the economic performance by 8-15% when compared to unconstrained RL). Securing a high level of safety comes with a significant trade-off in safe RL between the strength of the safety guarantees and economic performance: a higher guarantee strength (hard constraints, safety shields) entails a worse economic performance, and vice versa for the weaker guarantees (penalty methods, Lagrangian relaxation).

6.2. Multi-Agent Reinforcement Learning

MARL solves the control problem in multiple areas or subsystems, and each agent deals with a limited region using communication for coordination. Another critical issue related to the convergence of MARL training is the problem of policy oscillation or convergence to a policy with poor convergence characteristics caused by the non-stationarity of the multi-agent environment, as noted by Chen et al. [33], who reported only 40% convergence of MARL training runs to stable policies on the IEEE 39-bus system, with the remaining 60% of runs converging to a policy with a poor convergence characteristic. The communication overhead is quadratic, namely O(n²), in fully connected agent topologies but could be linear in the number of neighbours, k, of an agent, O(n·k), in the case of a graph-based communication protocol [34]. In practice, MARL can be trained in sub-100 ms effective periods, suffering from high training instability and without guaranteed convergence.

6.3. Existing Hierarchical Controls in Practice

The power industry has used hierarchical control structures for more than 50 years. Automatic generation control (AGC) is used to control the area control error (ACE) within the control areas with an operating time of 2–4 s [35]. The economic dispatch is a function that updates the commitment and generation allocation of units (applies OPF-based methods) every 5–15 min. At the supervisory level, known as SCADA/EMS, single measurements every 2-4 seconds and state estimation and contingency analysis on a 5-15 minute cycle [36]. Relay protection in action at the fastest level is within 1-5 cycles (17-83 milliseconds). This existing hierarchical structure is static by design, as the timescale separation of < 100 ms (protection), 2–4 s (AGC), and 5–15 min (economic dispatch) ensures that fast protection does not wait for slow optimisation. Conservatism is by design: a relay that waits for economic dispatch to trip would be disastrous. The drawback of this static allocation is that the system cannot dynamically shift the resources allocated to different time scales. When the system entered a cascading failure—the fast protection layer is overloaded and the slow optimization layer has out-of-date data—there is no way for the system to switch to an operating mode. It is not that there are no hierarchies, but that there is no lively arbitration between them.

D. Why Existing Hierarchies Are Static

There are three reasons for the static hierarchies to remain. First, Regulatory Inertia: Relays are certified to standards defined by IEEE C37 and are not programmable; they can only be altered through extensive testing and regulatory approval of the altered program[2]. Second, communication limitations: SCADA systems work on dedicated communication channels, usually with an update rate of 2-4 seconds, which is a fundamental limitation for any coordination mechanism based on centralised information [36]. Third, and most basic, is the lack of a theoretical model of dynamic mode arbitration: there is no clear theory for knowing when to switch between reflexive and deliberative control modes during real-time execution. This paradigm currently assumes event type/fault → protection, frequency deviation → AGC, and cost minimisation → OPF; however, it fails to hold true when compound events are executed and multiple timescales are in play simultaneously. For example, an event that causes a fault, which in turn causes frequency changes, requires economic re-dispatching within seconds.

Table 3 Quantitative comparison of hybrid and multi-agent methods

| Method | # Studies | Latency (mean±σ ms) | Cost Gap (% vs. OPF) | Constraint Violations | Empirical Validation | Scalability (max buses) | Quality Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Safe RL (CPO) | 5 | 20–150 (60±45) | 5–15% (9.2±3.8) | <0.5% (hard constr.) | Yes (sim) | 39 | Q2–Q3 |
| Shielded RL | 4 | 3–20 (8±6) | 8–20% (13.5±5.0) | 0% (shield enforced) | Yes (sim/HIL) | 39 | Q3 |
| Lagrangian RL | 6 | 10–80 (30±22) | 3–10% (6.1±2.5) | 0.5–3.0% (1.5±0.9) | Yes (sim) | 118 | Q2–Q3 |
| MARL (QMIX) | 5 | 100–300 (160±70) | 5–15% (8.5±3.6) | 1.0–5.0% (2.8±1.4) | Yes (sim) | 118 | Q2 |
| MARL (MAPPO) | 4 | 80–200 (120±50) | 3–12% (7.0±3.2) | 0.5–3.0% (1.8±0.9) | Yes (sim) | 39 | Q2 |
| Hierarchical RL (two-level) | 3 | 50–150 (85±40) | 4–10% (6.5±2.8) | 0.5–2.0% (1.2±0.6) | Yes (sim) | 39 | Q2–Q3 |
| AGC + OPF (deployed) | N/A (practice) | 2000–900000 (AGC: 2–4s; ED: 5–15min) | 0% (OPF) (suboptimal timing) | 0% (certified) | Field deployed | >10,000 | Q3 (field) |
| Statistical Summary | 27 total (+practice) | Median: 80 Mean: 110±120 | Median: 7.0% Mean: 8.3±4.5% | Median: 1.2% Mean: 1.7±1.4% | 27/27 (100%) | Max: 118 (>39 rare) | Q3: 8 Q2: 15 Q1: 4 |

AGC + OPF represents the current industrial practice and is not from the academic literature; it is included as a reference point for comparison. Scalability figures for academic methods represent the largest system on which the method has been validated in the reviewed studies; this is typically much smaller than the systems on which industrial methods operate.

7. Quantitative Gap Analysis: The Pareto Frontier

7.1. Latency-Optimality Pareto Frontier

Fig. 1 (conceptual) plots the latency-optimality Pareto frontier across all six paradigms in the study. The x-axis represents the median latency (log scale, ms), and the y-axis represents the median cost gap versus OPF-optimal (%; lower is better). The Pareto frontier is constructed by identifying methods that are not dominated by any other method in both dimensions simultaneously. The frontier consists of: (1) FCS-MPC at the extremely low-latency end (0.025–0.5 ms), which optimises local tracking variables but features no system-level economic cost metric; (2) neural network surrogates in the intermediate zone (3–8 ms, 3–12% cost gap); (3) PINNs (5–50 ms, 1–8% cost gap); (4) single-agent DRL methods (1–500 ms, 2–12% cost gap); and (5) classical OPF methods (50–2000 ms, 0% cost gap). Although Multi-Agent Reinforcement Learning (MARL) variants are evaluated, they are fundamentally sub-Pareto-optimal on this strict 2D plane; their coordination overhead (80–300 ms) causes them to be dominated by centralised optimisation on cost and by single-agent architectures on latency. The critical observation is that no single baseline method simultaneously and reliably occupies the target region of (<5 ms discrete latency, <3% cost gap) under nominal grid conditions while ensuring physical constraint compliance, which is the performance envelope mandatory for unified reflexive-deliberative operation. The 5–50 ms window remains particularly sparse: only neural surrogates and PINNs operate here, but both exhibit cost gaps above 3% and nonzero constraint violation rates.

Figure 1 Latency-optimality Pareto frontier analysis

Updated Table III: Quantitative Comparison of Hybrid and Multi-Agent Methods

Table 4 Quantitative comparison of hybrid and multi-agent methods

| Method | # Studies | Latency (mean ± σ, ms) | Cost Gap (% vs. OPF) | Constraint Violations | Empirical Validation | Scalability (max buses) | Quality Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Safe RL (CPO) | 5 | 20–150 (60 ± 45) | 5–15% (9.2 ± 3.8) | <0.5% (hard constraints) | Yes (simulation) | 39 | Q2–Q3 |
| Shielded RL | 4 | 3–20 (8 ± 6) | 8–20% (13.5 ± 5.0) | 0% (shield enforced) | Yes (simulation/HIL) | 39 | Q3 |
| Lagrangian RL | 6 | 10–80 (30 ± 22) | 3–10% (6.1 ± 2.5) | 0.5–3.0% (1.5 ± 0.9) | Yes (simulation) | 118 | Q2–Q3 |
| MARL (QMIX) | 5 | 100–300 (160 ± 70) | 5–15% (8.5 ± 3.6) | 1.0–5.0% (2.8 ± 1.4) | Yes (simulation) | 118 | Q2 |
| MARL (MAPPO) | 4 | 80–200 (120 ± 50) | 3–12% (7.0 ± 3.2) | 0.5–3.0% (1.8 ± 0.9) | Yes (simulation) | 39 | Q2 |
| Hierarchical RL (two-level) | 3 | 50–150 (85 ± 40) | 4–10% (6.5 ± 2.8) | 0.5–2.0% (1.2 ± 0.6) | Yes (simulation) | 39 | Q2–Q3 |
| AGC + OPF (deployed) | N/A (practice) | 2,000–900,000 (AGC: 2–4 s; ED: 5–15 min) | 0% (OPF) (suboptimal timing) | 0% (certified) | Field deployed | >10,000 | Q3 (field) |
| Statistical Summary | 27 total (+practice) | Median: 80; Mean: 110 ± 120 | Median: 7.0%; Mean: 8.3 ± 4.5% | Median: 1.2%; Mean: 1.7 ± 1.4% | 27/27 (100%) | Max: 118 (>39 rare) | Q3: 8, Q2: 15, Q1: 4 |

Figure 2 Latency distributions by paradigm and system size

7.2. There is no single method that dominates

The Pareto analysis shows that there is no existing model capable of satisfying the three important aspects of fast response, optimisation of cost, and satisfaction of constraints by a single unified algorithm with dynamic mode arbitration. Low latency (FCS-MPC, neural surrogates) does not include cost awareness and constraint guarantees. Latency cannot be satisfied for methods which are cost optimal (OPF). Approaches that seek a compromise between both (DRL, PINNs, safe RL) are placed in a mediocre middle ground, with cost deviations and violation rates of 2–12% and 0.5–5%, respectively. The gap is not only qualitative but also quantitative (in terms of degree) and qualitative (in terms of architectural capability); there is no existing way to dynamically switch coefficienting from reflexive to deliberative modes, depending on conditions as the system is running.

7.3 Central Gaps Section

7.3.1 Central Gap: No Dynamic Arbitration Mechanism

The empirical gap identified through the study has been established, with 127 works studying the consensus on the non-existence of a dynamic mode arbitration mechanism. Mode arbitration is formally specified as the logic governing which control mode (reflexive or deliberative) is in operation at any particular time instant. The quantitative analysis of the latency-optimality Pareto frontier shows that it is impossible to satisfy the three integral goals, namely latency under 5ms, cost optimality within 3% of the OPF-optimal, and hard constraints, simultaneously with the current practice. Static, preprogrammed arbitration is used in existing systems: protection relays initiate action based on a predetermined current level, optimised power flow (OPF) runs with set schedules in the 5–15 min range, and automatic voltage control (AVC) operates based on ACE deviation.

Formal Problem Statement: Let the power system state at time tbe denoted x(t)∈Rn, comprising bus voltage magnitudes, voltage angles, frequencies, and their time derivatives. Define the mode space S={reflexive,deliberative}and the arbitration function Mode:R+→Smapping time to the active control mode. The system dynamics are governed by the state-space equation

dxdt=f(x(t),u(t),m(t)),

where u(t)∈Rmis the control input and m(t)=Mode(t)∈Sis the active mode selected by the arbitration function.

The objective is to minimize the weighted cost

J=w1⋅τlatency+w2⋅cgap+w3⋅vviolations,

where τlatencyis the mode-switching latency, cgapis the cost gap versus OPF-optimal dispatch, vviolationsis the aggregate constraint violation magnitude, and w1,w2,w3are positive weights.

The optimization is subject to:

- (i) timing constraints τ<5ms for reflexive mode and τ<50ms for deliberative mode (derived from IEEE C37 and NERC standards in Section III);

- (ii) physical constraints including voltage limits (0.95–1.05 pu), frequency limits (59.5–60.5 Hz), and line flow limits;

- (iii) communication constraints including bandwidth and packet loss bounds.

Feasibility requires that the reflexive controller be guaranteed to satisfy all hard constraints under any mode transition, which is the formal statement of the certainty-equivalence breakdown discussed in Section VII-F. The arbitration function Mode(t)is the decision variable; its optimal design is the subject of Hypothesis H1.

This fragmentation is documented by a meta-analysis of 127 studies, including six control paradigms (FCS-MPC, neural networks, OPF, DRL, PINNs, and MARL). Reflexive methods(FCS-MPC, neural surrogates) attain median latencies of 2–15ms, but fail to provide any form of cost awareness (accounting for only 18.8% of reflexive studies), and no constraint guarantees. Deliberative methods (OPF, convex relaxations) guarantee a 0% optimality gap but have latencies of 50ms to over 1 s, which are far higher than the time required for fault clearing in relay protection (83ms, IEEE C37.013). The range from 5 to 50 ms is underpopulated; only PINNs and fast DRLs function in this time regime, each with a cost gap of 1–12% and constraint violation rates ranging from 0.5% to 5%.

This is not a straightforward technical deficiency; there is no logic of decision about changing modes depending on a condition. The reviewed literature shows no method that is able to (a) detect when a transient is correct and switch from reflexive to deliberative mode for an optimisation, (b) detect an outdated optimisation solution and reactively invoke reflexive fall-backs, or (c) detect an invalid model (topology change due to a fault) and adjust operating modes accordingly.

7.3.2 Contributing Gap 2: Constraint Internalization

Embedding nonlinear AC power flow constraints into learning-based controllers is a fundamentally unsolved problem, which has been validated empirically across 44 deliberative control studies. Barrier functions guarantee strict feasibility but are numerically ill-conditioned and introduce numerical instability, necessitating a gradient explosion to avoid reaching the constraint boundaries, thereby making the feasibility problem more difficult to solve. Convex relaxations (SDP or SOCP) theoretically guarantee feasibility, but show duality gaps of 1–5% for meshed transmission networks (median 1.4%, test-based according to the meta-analysis of 10 PINN studies), and solutions obtained for the relaxed version of the problem are not guaranteed for the original problem (AC) and only work if one has additional information that they are feasible.

One important result that ensued from the reviewed literature is that, although specifically tailored to encode physics constraints, PINN-based models still fail to guarantee hard constraint satisfaction. This indicates that the problem is not only algorithmic in nature, but it could be a fundamental inherent difficulty in encoding the non convex AC-Power flow constraints into neural network loss functions.

7.3.3 Contributing Gap 3: Scalability

The scalability crisis is documented quantitatively: out of 127 studies reviewed, only 23 (18.1%) report results on 118 buses or more, and none report validation on systems of more than 300 buses. The methods that work well with 9–39 bus systems do not scale to 118+ bus systems. The number of modes must be calculated, and for 10 controllable generators and 46 switchable lines, the 39-bus case would have a computational number of approximately 2^56 modes to be evaluated exhaustively (as in FCS-MPC) in any real time budget. At the core of Gap 1 is the latency problem, which is compounded by problems within the distributed techniques, e.g., 50-100ms documented in five reviewed MARL studies, which may overcome this scaling barrier. Computational and communication improvements not yet evidenced in the peer-reviewed literature are required to advance to more than 1,000 buses, the scale of real interconnections.

7.3.4 Contributing Gap 4: Cyber-Resilience of Arbitration Logic

Although the mode arbitration layer is necessary, it introduces a new attack surface. If a measurer can feed false values into the arbitration logic, he can cause an arbitrary mode transition on one of these occasions: (1) from normal operation to reflexive (protection) mode, which causes unnecessary disruption and economic loss, or (2) from a fault condition to normal operation, which causes damage to equipment and possible cascading failure. The adversarial example and sensor spoofing robustness of arbitration decision logic remain open problems. Although classical relay protection is inherently robust because of its simplicity, a learned arbitration policy (whether a neural network or reinforcement learning) has never been studied extensively for robustness to adversarial perturbations.

7.3.5 Contributing Gap 5: Self-Assessment Capability

The explicit, real-time self-assessment capabilities of controllers should allow for dynamic mode arbitration, a requirement for controllers that is found in the literature but has not been included in real applications. Self-assessment must be able to detect (a) model invalidity (topology change due to a fault or equipment outage), (b) measurement corruption (sensor loss, sensor drift, or cyber attack on sensor signals), (c) loss or latency degradation in communications, and (d) current operating mode and validity of the mode-dependent model assumptions. This is the adaptive control and fault detection problem, which has received considerable attention in control publications [37] but has not been tested on realistic test systems in real-time mode arbitration for power systems, as has been done in those studies. For decisions regarding the switching from the deliberative optimiser and degraded protection systems, a self-assessment layer that signifies the invalidity of the deliberative optimiser or degraded protection systems is necessary. Otherwise, the system cannot determine whether a switch is required.

7.3.6 Gap Dependencies and Interactions

The five gaps are not independent issues to be addressed in isolation but rather pieces of a single puzzle. Gap 1 (arbitration) depends on Gap 2 (internalisation of constraints). Because the arbitration logic must be aware of whether the current control mode meets the physical constraints to decide valid switching, the above must be satisfied. For the same reason, Gap 1 also relies on Gap 5 (self-assessment) to initiate a mode switch when the current mode fails or becomes invalid. Gap 3 (scalability) follows from Gap 1 (computational complexity of any sophisticated arbitration mechanism is prohibitive in large systems). However, distributed solutions that can help alleviate the complexity (such as using MARL using agents for each zone) cause coordination delays that aggravate the latency problem central to Gap 1. Gaps 1–3 and 4 are orthogonal, but Gap 4 is also closely connected with Gap 5: Cyber security mechanisms that prevent measurement contamination are themselves a type of cyber defense, but the logic of the self-assessment must be defended against measurement contamination. A critical interaction exists between Gaps 1 and 2, such that the computational budget devoted to making the arbitration logic faster or more accurate may be diverted from constraint enforcement, which is a fundamental resource allocation issue in any unified architecture.

Figure 3 Gap dependency network

7.3.7 Discussion of Fundamental Limits

The reflexive–deliberative gap may be partially a fundamental information-theoretic limit and partially an engineering limitation. Four rigorous results characterize this boundary.

Result 1 (Delay-Constrained Estimation Lower Bound). Under noisy measurements with communication delay Δ and signal-to-noise ratio SNR, the optimal estimate of the system state x(t)satisfies

E∥x(t)-x(t)∥2≥σ21+SNR,

where σ2is the prior variance [39]. This lower bound is tight for Gaussian channels and establishes that faster decisions (smaller Δ) necessarily operate with less information, while more informed decisions (larger Δ) are necessarily slower. For the 5 ms arbitration target (H1), this bound implies that the estimation error is at least σ2/(1+SNR)regardless of the algorithm used, providing a floor on the false-positive rate.

Result 2 (Certainty Equivalence Breakdown). The certainty equivalence principle [40] states that the optimal control under uncertainty equals the optimal control under certainty when the uncertain quantity is replaced by its expected value. However, this principle breaks down when the cost of suboptimal decisions is asymmetric. In power systems, over-tripping (unnecessary reflexive activation) incurs a dispatch cost penalty of 1–3% per event, while under-tripping (failure to activate reflexive mode during a fault) risks equipment damage of $100K–$1M per event. This asymmetry, documented in Section VIII-C, formally invalidates certainty equivalence and necessitates explicit mode arbitration rather than threshold-based switching.

Result 3 (Dual Control and Exploration–Exploitation). Dual control theory [41] establishes that optimal control under parameter uncertainty requires exploratory actions that are suboptimal in the short term but reduce uncertainty for future decisions. This creates an exploration–exploitation tension that directly parallels the reflexive–deliberative dichotomy: reflexive control exploits current knowledge for immediate response, while deliberative control explores the optimization landscape for better long-term solutions. The optimal balance, characterized by the Bar-Shalom covariance penalty, provides a principled basis for the arbitration function Mode(t)defined in Section VII-C.

Result 4 (Information-Theoretic Limit for 5 ms Arbitration). Combining Results 1–3, the information-theoretic lower bound on arbitration latency can be derived as follows. For a measurement channel with capacity Cbits/second and a state vector of dimension nrequiring Bbits per dimension for adequate representation, the minimum latency is

τmin≥n⋅BC.

For a typical 39-bus system with n=156state variables (3 per bus: voltage, angle, frequency), B=16bits per dimension, and a PMU channel capacity of C=4800bits/second (30 frames/second × 160 bits/frame), we obtain

τmin≥156×164800=0.52 seconds.

This suggests that sub-5 ms arbitration requires either dimensionality reduction (fewer state variables), lossy compression (fewer bits per dimension), or higher-bandwidth communication than standard PMU networks. The 5 ms target is achievable only with aggressive feature selection and high-speed communication, confirming that it is near the information-theoretic frontier.

Synthesis. These four results collectively demonstrate that the 5–50 ms gap is not merely a consequence of suboptimal algorithms but reflects a fundamental trade-off between information quality and response speed. However, the current state of the art operates far from the information-theoretic frontier, suggesting that significant engineering gains remain possible through better state compression, faster sensing architectures, and tighter coupling between estimation and control.

8. Arbitration Challenges: Latency-Accuracy Trade-Offs

8.1. Fault Detection vs. Transient Discrimination

The most pressing problem related to mode arbitration is distinguishing faults (which require an immediate reflexive response) from normal transients (which can be deliberated with normal operation continuing). This discrimination must occur on a millisecond timescale and be based only on local measurements. Conventional fault detection technologies, such as overcurrent, distance, and differential relaying, can detect faults with a delay ranging from 5-20ms, and a false alarm rate of less than 0.1% under tightly tuned operations [2]. However, these techniques are only for binary detection (fault/no fault) and are not suitable for the more nuanced assessment of mode arbitration (event severity, event duration, and the appropriate control response). Neural network detection methods can detect event types with 95–99% accuracy [38], 1–5% of which is a hit–and–miss type error rate or an economically more expensive false alarm rate. One case that is especially difficult is the discrimination between a true transient due to a fault and a large load switching turn-on event or a ramp start of renewables that produce fast frequency and voltage variations but call for very different control actions. Because the measurement rate in frames per second (or time lapses in milliseconds) is 30–60 fps (16–33 ms), synchro phasor-based wide-area measurement systems (WAMS) are too slow for sub-5 ms arbitration decisions.

8.2. Latency vs. Training Time

A speed/accuracy trade-off is inherent in detection. Lowering the threshold yields fewer missed faults but also generates more false alarms, whereas raising the threshold leads to fewer false alarms but potentially missing low-magnitude faults. The numbers in Table V illustrate this tradeoff with data from both relay protection studies and neural network classifiers.

Table 5 Detection threshold vs. false positive rate vs. missed fault rate

| Detection Threshold | Detection Latency (ms) | False Positive Rate (%) | Missed Fault Rate (%) | Appropriate Mode | Annual Cost Impact |
| --- | --- | --- | --- | --- | --- |
| Very aggressive (1σ deviation) | 1–2 | 15–25 | <0.1 | Reflexive (unnecessary 15–25%) | High: unnecessary trips |
| Aggressive (2σ deviation) | 2–5 | 3–5 | 0.1–0.5 | Reflexive (justified) | Moderate: occasional trips |
| Moderate (3σ deviation) | 5–10 | 0.3–1.0 | 0.5–2.0 | Arbitration (zone) | Low-moderate: delayed response |
| Conservative (4σ deviation) | 10–20 | <0.1 | 2–5 | Deliberative (risky for faults) | Low: missed faults |
| Very conservative (5σ deviation) | 20–50 | <0.01 | 5–10 | Deliberative (unsafe for faults) | Very low: catastrophic risk |

The optimum operating range for mode arbitration is the moderate zone (3σ threshold), which has a detection delay of < 10ms and false positive rates between 0.3% and 1.0%, and missed fault ratio < 2%. This threshold matches the 3 ± 1 ms arbitration target set forth in the IEEE C37 standards in section III. However, the actual threshold value depends on the ratio between the false-positive rate and fault-missed rate of the system and operating conditions.

8.3. Cost of Conservative vs. Aggressive Mode Switching

A high timeout and few mode switches (conservative switch) could be an exposition to exposure during a fault, resulting in equipment damage and cascading faults. Aggressive switching (low threshold, many switches) leads to many unnecessary reflexive-mode switchings which are safe but do not optimise the system and can result in oscillatory behaviour if the system rapidly switches between modes. We estimated the cost impact as follows: In a 39-bus system, a false positive (unnecessary reflexive activation) carries a dispatch cost penalty of approximately 1–3% per event, or 10–50 such events/year on a typical system[39]. The expected cost of a missed fault, which is a failure to automatically switch to reflexive mode, is much higher: the cost of equipment failure ($100K-$1 million per miss) times the chance of cascading failure (0.01-0.1 per miss) equals the expected cost per annual miss event of $1K-$100K per year. This one-sidedness favours aggressive switching; it is preferable to act reflexively; however, the cost/benefit relationship must be determined for each system.

8.4. Physics Constraint Internalization Feasibility

Embedding a nonlinear AC power flow constraint into neural network controllers is a prerequisite for constraint-aware arbitration. There are three approaches, each with its own limitations. Convex relaxations (SDP, SOCP) provide hard constraint guarantees but can be inexact for meshed networks, which may provide feasible solutions to a relaxed problem but not to the original AC problem[23], [24]. Second, penalty methods are flexible and can incorporate arbitrary nonlinear constraint functions which are added to the loss function with tunable penalty coefficients, but it does not guarantee that there is zero constraint violation.[29]. Third, barrier functions impose strict feasibility conditions such that the cost function is made infinite in the proximity of constraint boundaries, but results in numeric ill-conditioning that is difficult to deal with during training and can lead to gradient explosion. A fourth method not discussed in the power system literature is projection-based methods which map the outputs of the neural network onto the feasible set determined by the power flow solver at every inference step. This ensures feasibility but at the cost of an iterative projection step with 5-50ms of latency per inference, which sometimes could outweigh the speed benefit of neural inference. The best approach would be to use penalty-based training (for approximate feasibility) and runtime projection (for hard guarantees) on top of it, with only a penalty if any constraint violations are detected by the self-assessment module.

9. Research Hypotheses

Based on the quantitative gap analysis and Pareto frontier characterisation, three research hypotheses were formulated to represent testable statements. The hypotheses define tests, fault types, success measurements, comparison baselines, and falsification criteria. They are research directions, not predictors of outcomes, and are stated as testable and falsifiable research directions.

9.1. Hypothesis 1: Dynamic Mode Arbitration

Configuring H1 as a mode arbitration layer that uses residual-based fault detection achieves an average false-positive rate lower than 5% and can support a fault-to-reflexive-mode latency of less than 3 ± 1 ms measured from the time the fault was detected on the IEEE 39-bus system under the following NERC TPL-001 contingency categories: single line to ground, double line to ground, and three-phase faults.

IEEE 39 bus – 10 generator system with standard dynamic models. The fault types were SLG (70% of the test cases), DLG (20%), and TLG (10%), and the fault locations were random at 10%, 50%, and 90% of the length of each of the 46 transmission lines. The success criteria were as follows: (a) fault-to-reflexive latency of 4ms or less for 95% of the test scenarios, (b) reflexive-to-deliberative latency ≤ 45ms or less for 95% of the test scenarios, and (c) false-positive rate ≤ 5% for 10,000 test scenarios. Comparison baselines: (a) static hierarchy (current practice), (b) threshold-based switching (if-then rules), and (c) no arbitration (reflexive-only and deliberative-only). Falsification criterion: The three measures of success are not achieved in at least three independent simulation runs using different random seeds, then H1 is falsified.

Sensitivity analysis: H1 should be tested for system sizes (IEEE 9-bus, 39-bus, and 118-bus) and varying renewable penetration (0%, 20%, 50%, and 80% of generations from inverter-based resources). The target latency of 3 ± 1ms is expected to hold up well at 0-50% penetration but could be impacted at 80% penetration because there is more relaxation in latency owing to the reduction in system inertia and quicker frequency dynamics, changing to 5 ± 2ms. The false-positive rate is expected to increase as a function of the system size (from 2% at 9-bus to 8% at 118-bus) because of the increased measurement space and more complex dynamic interactions.

9.2. Hypothesis 2: Physics Constraint Internalization

The rates of constraint violation (voltage excursion from the target levels above or below +/-0.05 pu for more than one cycle, frequency deviation from the target levels above or below +/-0.5 Hz for more than ten cycles, generator output excursion from target levels above or below its rated levels) are less than 0.2% with penalty-based loss function embedding physics constraints under training, while dispatch cost is within 5% of optimal dispatch cost on the IEEE 39-bus system achieved by MATPOWER OPF.

Test system: IEEE 39-bus with time-varying load profiles (24-hour horizon, 5-minute resolution) and renewable generation profiles at 20%, 50%, and 80% penetration. Constraint Definitions are explicit and measurable: violations of voltage constraints at each bus, frequency constraints measured at the Cen tenor of inertia, and generator limit violations are measured at each unit. The success metrics are as follows: (a) violation rates must not exceed 0.2% of all bus-time-step pairs; (b) dispatch cost should not exceed 1.05 × MATPOWER OPF cost; or (c) the maximum violation magnitude cannot exceed 0.1 pu (voltage), 1.0 Hz (frequency), or 10% above rated (generation). That is, (a) an unconstrained neural network surrogate, (b) hard-constrained optimisation (OPF), and (c) shielded RL. To falsify H2, if the violation rate is > 0.5% or the cost gap is > 10% in any of the tests, then H2 is falsified.

Sensitivity analysis: penalty coefficients (λ = 0.1, 1.0, 10.0, and 100.0), constraint types (voltage-only, frequency-only, and combined), and system sizes (9-bus, 39-bus, and 118-bus). Penalty coefficients will likely lead to a drop in violations and a corresponding rise in the cost gap based on the conservative nature of the policy. However, the 0.2% violation target is likely to be achieved in λ = 10–100 in the 39-bus system, but it may be necessary to increase the value of λ to > 100 in the 118-bus system, for which the state space is larger, making constraint satisfaction more difficult.

9.3. Hypothesis 3: Cost improvement due to Unified Architecture

A unified reflexive-deliberative architecture with dynamic arbitration can achieve 45–55% cost reduction compared to a baseline of independent reflexive controller (relay protection) plus independent deliberative controller (hourly OPF) running in parallel on the IEEE 39-bus system at 20–50% renewable penetration levels.

The cost metric is defined as the total system operating cost over a 24-hour simulation horizon, including the generation fuel cost, load shedding penalty ($1000/MWh), and renewable curtailment cost ($50/MWh). This is a baseline, a feasible possibility that currently exists in the industry: relay protection with fault response (reflexive) in parallel with OPF for economic dispatch (deliberative) running hourly. The improvement is derived from the flexibility in dynamic arbitration, which allows for quicker re-optimisation following fault events ( shortens load shed durations), co-optimisation of protection and dispatch ( reduces unnecessary curtailment), and adaptive behaviour in response to the variability of renewables ( reduces the requirement for reserves). It is a fair comparison: the unified architecture is compared against - not against a straw-man of purely reflexive operation–the status quo of two independent controllers. We recognise that the range of 45–55% is highly sensitive to the assumed cost of load shedding and renewable curtailment, as the improvement would range between 20% and 30% at lower load shedding costs ($100/MWh). Success criteria: (a) costs must be reduced by at least 45% at 20-50% penetration, (b) no rise in the constraint violation rate above a baseline value, and (c) cost reduction must be monotonic with increasing penetration (i.e. increasing penetration translates into increasing cost benefit from fast re-optimisation, indicating more value to such fast re-optimisation). Comparison baselines: (a) relay + hourly OPF (status quo), (b) relay + 5-minute OPF (enhanced status quo), (c) reflexive-only, and (d) deliberative-only. Falsification criterion: H3 is falsified if either the cost improvement is less than 30 per cent at a penetration of 20–50 per cent or if there are more constraint violations than in the base case.

Caution: The statement of improvement of 45- to 55% is indicative and assumes a baseline without adaptive coordination between the reflexive and deliberative components. A better comparison is the enhanced status quo (relay + 5-minute OPF) which would bring the expected increase down to 15-25%. Both comparisons are reported; practical improvement is dependent on the regulatory and operational environment of the system.

Table 6 Sensitivity matrix for hypotheses 1–3

| Parameter | H1: Arbitration Latency | H1: False Positive Rate | H2: Violation Rate | H2: Cost Gap | H3: Cost Improvement |
| --- | --- | --- | --- | --- | --- |
| 9-bus system | 2±0.5 ms | 2% | 0.1% | 3% | 35–45% |
| 39-bus system | 3±1 ms | 5% | 0.2% | 5% | 45–55% |
| 118-bus system | 5±2 ms | 8% | 0.5–1.0% | 8–12% | 25–35% |
| 0% renewables | 3±1 ms | 3% | 0.1% | 3% | 20–30% |
| 20% renewables | 3±1 ms | 4% | 0.2% | 5% | 45–55% |
| 50% renewables | 4±1.5 ms | 6% | 0.3% | 7% | 50–60% |
| 80% renewables | 5±2 ms | 8% | 0.5–1.0% | 10–15% | 55–65% |
| SLG fault | 3±1 ms | 3% | — | — | — |
| DLG fault | 3.5±1.5 ms | 5% | — | — | — |
| TLG fault | 4±2 ms | 7% | — | — | — |

Figure 4 Hypothesis algorithms

10. Conclusion and Research Roadmap

In this review, the quantitatively defined latency-optimality Pareto frontier, spanning six power system control paradigms, shows that the frontier is a disjoint set; no single paradigm is optimal for all five review dimensions (latency, cost optimality, constraint compliance, scalability, and self-assessment). The central research gap is the lack of a mechanism to detect, in real time, whether reflexive or deliberative control is prevailing. The four gaps contributing to this central gap (constraint internalisation, scalability, cyber-resilience, and self-assessment) must be tackled in tandem, rather than sequentially, with clear dependencies and interactions between them.

We have formulated three falsifiable research hypotheses addressing the most critical aspects of this gap: (H1) dynamic mode arbitration with less than 5 ms fault-to-reflexive latency and less than 50 ms reflexive-to-deliberative latency, (H2) physics constraint internalisation, with less than 0.2% violation rate and less than 5% cost gap, and (H3) 45–55% cost improvement by going from independent reflexive plus deliberative controllers to unified architecture. Specific test systems, fault types, success criteria, comparison baselines, and means to falsify each hypothesis were provided to make future work testable rather than assuming it.

These are areas for research, not outcomes that are absolutely assured. Although the 5–50 ms range is near the Pareto frontier, existing approaches (PINNs and fast DRL) provide partial solutions but with significant limitations. It is important to recognise that the engineering limitation or the fundamental information-theoretic limitation will distinguish between the two: if the engineering limitation, then principled algorithmic and hardware solutions to the limitation are needed; fundamental (information-theoretic) limitation, then the research community must develop principled methods for operating under the fundamental limitation. Delay-constrained optimisation, certainty equivalence, and dual control theory, as presented and analysed here, are far from information-theoretic limits, presenting many opportunities for engineering enhancements.

This review has several limitations. First, the reported results for each study are limited to and from the individual studies and may be based on different simulation software, solver configurations, and validation criteria; hence, direct numerical comparisons should be made with caution. Second, the quality scoring system is transparent but is necessarily coarse-grained considering that a 300-bus system with a high-fidelity study has the same weightage as a 9-bus system with a study carried out with a simplified model. Third, the publication effects of our sensitivity analysis eliminating Q1 studies are not completely eliminated; however, we should not ignore the fact that the analysed studies might have been more (or less) optimistic than they truly were because other studies that resulted in negative or inconclusive findings were not recorded. Fourth, hypothesised performance targets (for example, arbitration latency within 3 ± 1 milliseconds) are not validated by experiments, and they are expectations to test (not claims to accept).

The research roadmap derived from this review is as follows: (1) develop and validate the mode arbitration layer (H1) as this is a prerequisite to the architecture necessary for unified control; (2) further develop the physics constraint internalisation (H2) concept to assist with effectively making switching decisions with constraints; (3) quantify and demonstrate the economic benefits of using unified control (H3) to encourage industrial use. Gaps 4 (cyber-resilience) and 5 (self-assessment) are orthogonal to the arbitration-constraint chain but are critical to the deployment of a real vehicle. Constraints are important if the controller cannot be arbitrated perfectly but very important if the controller cannot detect cyber-attacks or sensor failures. We welcome and encourage all members of the power system and machine learning communities to test these hypotheses, replicate and/or challenge them, and move the field closer to reflexive-deliberative models that are both safe and efficient across the entire power system time tables.

References:

[1]	P. Kundur and O. P. Malik, Power system stability and control, Second edition. New York: McGraw Hill Education, 2022.

[2]	“IEEE Std C37.013-1997 Revision of IEEE Std C37.013-1993 IEEE Standard for AC High-Voltage Generator Circuit Breakers Rated on a Symmetrical Current Basis”, [Online]. Available: http://rfc.nop.hu/ieee2/IEEE%20Std%20C37.013-1997,%20IEEE%20Standard%20for%20AC%20High-Voltage%20Generator%20Circuit%20Breakers%20Rated%20on%20a%20Symmetrical%20Current%20Basis.pdf

[3]	R. D. Zimmerman, C. E. Murillo-Sanchez, and R. J. Thomas, “MATPOWER: Steady-State Operations, Planning, and Analysis Tools for Power Systems Research and Education,” IEEE Trans. Power Syst., vol. 26, no. 1, pp. 12–19, Feb. 2011, doi: 10.1109/TPWRS.2010.2051168.

[4]	S. Vazquez, J. Rodriguez, M. Rivera, L. G. Franquelo, and M. Norambuena, “Model Predictive Control for Power Converters and Drives: Advances and Trends,” IEEE Trans. Ind. Electron., vol. 64, no. 2, pp. 935–947, Feb. 2017, doi: 10.1109/TIE.2016.2625238.

[5]	M. Glavic, R. Fonteneau, and D. Ernst, “Reinforcement Learning for Electric Power System Decision and Control: Past Considerations and Perspectives,” IFAC-Pap., vol. 50, no. 1, pp. 6918–6927, Jul. 2017, doi: 10.1016/j.ifacol.2017.08.1217.

[6]	M. Raissi, P. Perdikaris, and G. E. Karniadakis, “Physics-informed neural networks: A deep learning framework for solving forward and inverse problems involving nonlinear partial differential equations,” J. Comput. Phys., vol. 378, pp. 686–707, Feb. 2019, doi: 10.1016/j.jcp.2018.10.045.

[7]	D. Moher, A. Liberati, J. Tetzlaff, D. G. Altman, and The PRISMA Group, “Preferred Reporting Items for Systematic Reviews and Meta-Analyses: The PRISMA Statement,” PLoS Med., vol. 6, no. 7, p. e1000097, Jul. 2009, doi: 10.1371/journal.pmed.1000097.

[8]	NERC, “Reliability Standard Audit Worksheet    EOP-003-2 – Load Shedding Plans.” [Online]. Available: https://nercipedia.com/inactive-standards/eop-003-2-load-shedding-plans/

[9]	NERC, “Transmission System Planning Performance Requirements  - TPL 001 - 5 - 1.” [Online]. Available: https://www.nerc.com/globalassets/standards/reliability-standards/tpl/tpl-001-5.1.pdf

[10]	ENTSO-E, “P1 – Policy 1: Load-Frequency Control and Performance.” [Online]. Available: https://eepublicdownloads.entsoe.eu/clean-documents/pre2015/publications/entsoe/Operation_Handbook/Policy_1_final.pdf

[11]	ERCOT, “Current Nodal Operating Guides.” [Online]. Available: https://www.ercot.com/mktrules/guides/noperating/current

[12]	nationalgridESO, “Guidance Notes for Power Park Modules GB Users - Issue 5.” [Online]. Available: https://www.neso.energy/document/202461/download

[13]	IEEE, “IEEE PSRC Working Group Report 1 PROTECTIVE RELAYING AND POWER QUALITY.” [Online]. Available: https://www.pes-psrc.org/kb/report/101.pdf

[14]	P. Karamanakos, T. Geyer, and S. Manias, “Direct Voltage Control of DC–DC Boost Converters Using Enumeration-Based Model Predictive Control,” IEEE Trans. Power Electron., vol. 29, no. 2, pp. 968–978, Feb. 2014, doi: 10.1109/TPEL.2013.2256370.

[15]	A. Bemporad and M. Morari, “Robust model predictive control: A survey,” in Robustness in identification and control, vol. 245, A. Garulli and A. Tesi, Eds., in Lecture Notes in Control and Information Sciences, vol. 245. , London: Springer London, 1999, pp. 207–226. doi: 10.1007/BFb0109870.

[16]	J. L. Bosmediano, A. Aguila Téllez, and R. A. O. Martínez, “Optimal Distribution Network Reconfiguration with Renewable Generation Using a Hybrid Quantum–Classical QAOA for Power Loss Minimization,” Energies, vol. 19, no. 5, p. 1148, Feb. 2026, doi: 10.3390/en19051148.

[17]	Z. Yi et al., “Finite-Control-Set Model Predictive Control (FCS-MPC) for Islanded Hybrid Microgrids,” in 2018 IEEE Power & Energy Society General Meeting (PESGM), Portland, OR, USA: IEEE, Aug. 2018, pp. 1–5. doi: 10.1109/PESGM.2018.8586292.

[18]	Y. Luo, M. Jing, T. Ji, F. Sun, and H. Liu, “A Robust Tube-Based Smooth-MPC for Robot Manipulator Planning,” Mar. 17, 2021, arXiv: arXiv:2103.09693. doi: 10.48550/arXiv.2103.09693.

[19]	B. H. C. and I. Jeena Jacob, “A Hybrid CNN-LSTM Attention-Based Deep Learning Model for Stock Price Prediction Using Technical Indicators,” Eng. Technol. Appl. Sci. Res., vol. 15, no. 5, pp. 28012–28017, Oct. 2025, doi: 10.48084/etasr.12685.

[20]	A. M. Nakiganda and S. Chatzivasileiadis, “Graph Neural Networks for Fast Contingency Analysis of Power Systems,” 2023, arXiv. doi: 10.48550/ARXIV.2310.04213.

[21]	X. Pan, T. Zhao, and M. Chen, “DeepOPF: Deep Neural Network for DC Optimal Power Flow,” 2019, arXiv. doi: 10.48550/ARXIV.1905.04479.

[22]	F. Capitanescu et al., “State-of-the-art, challenges, and future trends in security constrained optimal power flow,” Electr. Power Syst. Res., vol. 81, no. 8, pp. 1731–1741, Aug. 2011, doi: 10.1016/j.epsr.2011.04.003.

[23]	J. Lavaei and S. H. Low, “Zero Duality Gap in Optimal Power Flow Problem,” IEEE Trans. Power Syst., vol. 27, no. 1, pp. 92–107, Feb. 2012, doi: 10.1109/TPWRS.2011.2160974.

[24]	M. Farivar and S. H. Low, “Branch Flow Model: Relaxations and Convexification (Parts I, II),” 2012, arXiv. doi: 10.48550/ARXIV.1204.4865.

[25]	B. Kocuk, S. S. Dey, and X. A. Sun, “Strong SOCP Relaxations for the Optimal Power Flow Problem,” 2015, doi: 10.48550/ARXIV.1504.06770.

[26]	J. Li, R. Zhang, H. Wang, Z. Liu, H. Lai, and Y. Zhang, “Deep Reinforcement Learning for Voltage Control and Renewable Accommodation Using Spatial-Temporal Graph Information,” IEEE Trans. Sustain. Energy, vol. 15, no. 1, pp. 249–262, Jan. 2024, doi: 10.1109/TSTE.2023.3282187.

[27]	A. Rizki, A. Touil, A. Echchatbi, R. Oucheikh, and M. Ahlaqqach, “A Reinforcement Learning-Based Proximal Policy Optimization Approach to Solve the Economic Dispatch Problem,” in The 1st International Conference on Smart Management in Industrial and Logistics Engineering (SMILE 2025), MDPI, Jun. 2025, p. 24. doi: 10.3390/engproc2025097024.

[28]	C. Qiang, H. Zhang, and A. Zhang, “Deep Reinforcement Learning Based Dynamic Pricing Demand Response of Smart Grid,” in 2022 First International Conference on Cyber-Energy Systems and Intelligent Energy (ICCSIE), Shenyang, China: IEEE, Jan. 2023, pp. 1–6. doi: 10.1109/ICCSIE55183.2023.10175274.

[29]	P. L. Donti, D. Rolnick, and J. Z. Kolter, “DC3: A learning method for optimization with hard constraints,” 2021, doi: 10.48550/ARXIV.2104.12225.

[30]	A. Venzke, G. Qu, S. Low, and S. Chatzivasileiadis, “Learning Optimal Power Flow: Worst-Case Guarantees for Neural Networks,” 2020, arXiv. doi: 10.48550/ARXIV.2006.11029.

[31]	G. Dalal, K. Dvijotham, M. Vecerik, T. Hester, C. Paduraru, and Y. Tassa, “Safe Exploration in Continuous Action Spaces,” 2018, arXiv. doi: 10.48550/ARXIV.1801.08757.

[32]	R. Cheng, G. Orosz, R. M. Murray, and J. W. Burdick, “End-to-End Safe Reinforcement Learning through Barrier Functions for Safety-Critical Continuous Control Tasks,” 2019, arXiv. doi: 10.48550/ARXIV.1903.08792.

[33]	D. Chen, K. C. Z. Li, T. Chu, R. Yao, F. Qiu, and K. Lin, “PowerNet: Multi-agent Deep Reinforcement Learning for Scalable Powergrid Control,” Aug. 01, 2021, arXiv: arXiv:2011.12354. doi: 10.48550/arXiv.2011.12354.

[34]	J. K. Gupta, M. Egorov, and M. Kochenderfer, “Cooperative Multi-agent Control Using Deep Reinforcement Learning,” in Autonomous Agents and Multiagent Systems, vol. 10642, G. Sukthankar and J. A. Rodriguez-Aguilar, Eds., in Lecture Notes in Computer Science, vol. 10642. , Cham: Springer International Publishing, 2017, pp. 66–83. doi: 10.1007/978-3-319-71682-4_5.

[35]	N. Jaleeli, L. S. VanSlyck, D. N. Ewart, L. H. Fink, and A. G. Hoffmann, “Understanding automatic generation control,” IEEE Trans. Power Syst., vol. 7, no. 3, pp. 1106–1122, Aug. 1992, doi: 10.1109/59.207324.

[36]	A. Bose, “Smart Transmission Grid Applications and Their Supporting Infrastructure,” IEEE Trans. Smart Grid, vol. 1, no. 1, pp. 11–19, Jun. 2010, doi: 10.1109/TSG.2010.2044899.

[37]	Y. K. Tan, S. Mitsch, and A. Platzer, “Verifying Switched System Stability With Logic,” in 25th ACM International Conference on Hybrid Systems: Computation and Control, May 2022, pp. 1–11. doi: 10.1145/3501710.3519541.

[38]	M. Coşkun, O. Polat, F. Doğan, and M. Aktaş, “A Hybrid CNN–LSTM Model with Attention Mechanism for Photovoltaic Power Estimation,” 2025.

[39]	A. G. Phadke and J. S. Thorp, Synchronized Phasor Measurements and Their Applications. in Power Electronics and Power Systems. Boston, MA: Springer US, 2008. doi: 10.1007/978-0-387-76537-2.